Hirox Digital Microscope KH-7700, Lens MX – 2016Z, Adapter AD – 2016S.

Measure > Calibration > Select from MAG LENS LIST

[Put asterix on left side onto correct lens] APPLY, CLOSE.

Measure > 2D MEASUREMENT

Linear measurement (Do not check ‘RATIO’)

Length = click once and measure LONGEST axis and click end.

Width = repeat for shorter axis.

To delete a measurement use the Dustbin button.

AREA

Check area box only.

Click repeatedly around object perimeter and double click to finish.

SAVE MEASUREMENTS (µm are appropriate)

Files are .csv format

Blue cross button to close measurements.

LIBRARY>LIBRARY>to copy files to USB or CD.